| Register | Default | Description |
| --- | --- | --- |
| 1 | 1 | Firing Mode: - Semi - Auto Response - Full Auto - Smooth Ramp (debounce slowly drops) - Assisted Ramp (turbo style ramping where it adds shots based on the rate of fire. Slower trigger pulls add less shots. Faster trigger pulls will add more shots. - Fast Ramping (Shots are added as soon as you reach the settings in register 5 and 6) - Triplet Shot Ramping (fires 3 shots every time the trigger is pulled. This speeds up the faster you pull the trigger) - PSP1 (3 shots then fast) (no need to set register 5/6 as these features are built in) - PSP2 (3 shots then super-fast ramping) (no need to set register 5/6 as these features are built in) - PSP3 w/Triple Shot ramping (3 shots thenfires 3 shots every trigger pull) - NXL (3 shots then full auto) - Breakout (Full Auto then fast ramping. Settings in registers 5/6 are honored) - OMFG (cant really explain it but it is amazing) WARNING: Don’t stand behind your own players when using this one. |
| 2 | 18 | Rate of Fire: This is the GLOBAL rate of fire. This controls the MROF regardless if the eyes are on or off. |
| 3 | 10 | Debounce: A higher setting will keep the gun from firing extra shots with each trigger pull. |
| 4 | 12 | Dwell: Bolt Forward Duration. Length of time the bolt stays forward |
| 5 | 4 | AFA ROF: 1 = off, 2 and above is the rate of fire that must be reached before the advanced firing modes activate. |
| 6 | 3 | AFA Shot Count: Actual number of trigger pulls before ramping activates |
| 7 | 15 | Fire Hold Off: Delay before the gun will fire again after cycling in ms |
| 8 | 1 | Eye Hold Off: Delay before the gun will fire after seeing a ball in ms. If you are using a slow hopper it might be necessary to increase this to avoid chopping. |
| 9 | 1 | Anti Bolt Stick: This is the time the board waits before increasing the dwell to compensate for a gun that has an issue with bolt stick. NOTE: Using this feature on a gun without this issue will result in the first shot being hotter. 1 = off 2 = 5s 3 = 10s 4 = 15s |
| 10 | 5 | Anti Bolt Stick Time: This is the increase in dwell when register 8 is engaged. |
| 11 | 11 | Eye Off Rate Of Fire: 1 = rate set in register 2, 2 and above equals the rate of fire that is needed when the eyes are turned off. |
| 12 | 1 | ROF Additions in .20 of a second: 1 = off, 2 = .20, 3 = .40, 4 = .60, 5 = .80 |
| 13 | 1 | Disable Eye thru trigger: 1 = yes, 2 = no (this feature allows you to turn the eyes off by holding the trigger back for 2 seconds) |
| 14 | 1 | Clearing Shot: 1 = yes, 2 = no (This allows you to fire a ball if you hold the trigger back for ¾ of a second in the event the eyes do not see a ball) |
| 15 | 5 | Eye Type: 1 = Break Beam, 2-20 = Reflective with sensitivity control) 2 = most sensitive 20 being the least. YES we can see black paint with our new reflective eye logic |
| 16 |  | Accessory Run Time (for boards that use this feature) |
| 17 |  | Reserved for future use |
| 18 |  | Reserved for future use |
| 19 |  | Reserved for future use |
| 20 |  | Reserved for future use |

Programming advanced firing mode example on the Predator SFT board:

Player wishes his or her shocker to ramp to the maximum speed once they reach 6bps.

- Go into programming mode by turning your Shocker on while holding the trigger back

- LED will flash once to indicate you are in program mode and will stay dim

- Pull trigger 1 time to enter the AFA register (1)

- LED will flash the current setting

- Pull the trigger 6 times to select mode 6 (fast ramping)

- LED will flash 2 times to indicate it took the new setting

- Pull trigger 5 times to go to register 5

- LED will flash the current setting

- Pull the trigger 6 times to indicate you wish the ramping to start at 6bps

- LED will flash 2 times to indicate it took the new setting.

- Turn the gun off and back on to use the new settings.

LED Operation and Functionality:

LED On = Ball in breech LED Slow Flashing = No ball in breech LED Fast Flashing = Eye off Led Dim = Program mode

UPDATE: Dip Switch 1 is your eye sensitivity control when using break beam eyes.  Down (off) is medium power for the emitter while Up (on) is full power for the reflective eye system.

To turn the eyes off during play simply tap the on/off button on the back of the frame and the light will quickly flash to indicate that the eye has been deactivated.  You can also turn the eyes off by holding the trigger back for 2 seconds.

Programming: ALL boards are shipped with the PROGRAMMING LOCK turned on unless we forgot to do it. This is highly possible so make sure you check it.

To program the Predator the tourney lock must be off (DIP switch 2 on the board) then pull and hold the trigger, turn the power on and release the trigger. The gun will fire with the tournament lock on and off.  However, while the tournament lock is activated you cannot move through the registers.  We strongly suggest you turn the tournament lock on while playing.

The light will flash bright blue then turn to a “dim” blue. Now you are expected to select a register as listed above. Click the trigger an appropriate number of times, then wait, the light will flash what that current registers setting is. After this you are expected to enter a register.

Example for setting dwell:

1. Pull and hold the trigger then turn the gun on by using the button on the back of the gun.

2. Release trigger when the light flashes once then goes dim.

3. Pull trigger 4 times to go to register 4(dwell).

4. Led will flash the current setting.

5. You can now pull the trigger to insert the new dwell setting.  15 trigger pulls = 15ms dwell

When you are finished setting the registers turn the gun off and back on to use the new settings.

NOTE: If you select Register 1, you are expected to enter a fire mode, again in trigger clicks. After you have made a selection, the light will flash and remain on. The gun is now prepared to fire.  We suggest you make the fire mode the last register you select as the board will reboot after it has been selected.

Function and Operation:

Pushing and holding the button on the rear of your grip frame will activate the shocker.

The gun will go into a sleep mode after 12 minutes of non-use.  To wake the gun, simply pull the trigger or hit the power button.

Warranty and Code Upgrades: Please visit our website for warranty and upgrade info